GridSenpAI Data Schema Documentation

Platform-agnostic architecture with example IBM implementation references in parentheses

# 1. Purpose

This document defines the structured schemas used by GridSenpAI to ingest facility engineering data, manage intermediate processing artifacts, and produce transmission-planning-ready model outputs. It is aligned to the current SRS, DSD/DCD, DMD, RTM, and TPVD artifact set.

# 2. Schema Design Principles

Schema-first contracts govern all service boundaries.

Unknown values remain null or omitted and must never be fabricated.

Every exported parameter must include provenance and confidence metadata.

Intermediate run artifacts must remain reconstructable through run lineage.

Implementations may use enterprise ingestion, search, and model platforms (for example IBM Watson Discovery and watsonx.ai), but the schema design remains platform-agnostic.

# 3. Core Processing Entities

| Element | Description |
| --- | --- |
| Project | Top-level workspace and access boundary for a facility evaluation. |
| Artifact | Submitted engineering document or file reference used during analysis. |
| IndexChunk | Searchable document segment derived from an artifact for retrieval operations. |
| Run | A single execution of the processing pipeline with pinned configuration and lineage. |
| RunConfig | Pinned schema, retrieval, model, and feature configuration for a run. |
| NormalizedInput | Schema-validated structured representation of extracted engineering inputs. |
| EvidenceSnippet | Retrieved evidence record used to support modeling parameter generation. |
| ModelOutput | Schema-valid output object containing generated planning parameters. |
| OutputParameter | Individual exported parameter with provenance and confidence metadata. |
| ScenarioSet | Typical, Conservative, and Best-Case scenario variants generated for a run. |
| Export | Packaged JSON/report deliverable set produced for downstream use. |

# 4. Input Schema

Input data may arrive through structured forms, APIs, or document extraction workflows. Document ingestion and search may be implemented with enterprise retrieval platforms (for example IBM Watson Discovery).

## 4.1 Core Input Categories

| Element | Description |
| --- | --- |
| facility_info | POI voltage, service configuration, site identifier, and phased buildout schedule. |
| one_line_diagram | File references for MV/LV one-lines, substations, and related electrical topology documents. |
| load_composition | IT load, cooling load, and other facility load fractions or schedules. |
| ups_systems | Vendor, model, topology, operating mode, module count, and rated capacity. |
| generators | Vendor, model, quantity, ratings, and operating/export capability. |
| transformers | Ratings, voltages, percent impedance, tap information, and connection details where available. |
| protection_settings | Protection device configuration, curves, ride-through, and relay settings summaries. |
| harmonics | THD or harmonic risk information, filter presence, and major power-electronics sources. |
| commissioning_constraints | Energization, sequencing, restrictions, and commissioning notes if available. |

## 4.2 Validation Rules

All power values must declare units and normalize to MW/Mvar for downstream outputs.

Voltage values normalize to kV.

Time/curve data should be expressed as structured time-value or per-unit/value pairs.

Missing critical data must trigger follow-up question generation rather than silent inference.

Schema validation must run before retrieval, translation, and export.

# 5. Intermediate Processing Schema

| Element | Description |
| --- | --- |
| ExtractionResult | Structured extraction artifact containing identified equipment, ratings, and topology cues. |
| ValidationReport | Errors, warnings, conflicts, and missing fields generated during normalization. |
| FollowUpQuestion | Structured clarification request tied to a schema field path and evidence context. |
| EvidenceBundle | Run-scoped collection of retrieved evidence snippets and retrieval metadata. |
| RunExecutionLineage | Pinned lineage metadata linking run, configuration, retrieval context, and model-assisted artifacts. |
| CanonicalStateSnapshot | Immutable authoritative state snapshot used for governed model-assisted processing where applicable. |
| PromptBuildRecord | Persisted prompt-construction metadata for governed model-assisted tasks where applicable. |

# 6. Output Schema

Outputs are generated by translation services using deterministic rules and, where appropriate, model-assisted extraction (for example watsonx.ai). Outputs must conform to the canonical output schema and be suitable for transmission planning use.

## 6.1 Output Categories

| Element | Description |
| --- | --- |
| steady_state | Pmax, Pmin, Q limits, PF targets, control mode, and related steady-state behavior. |
| zip_model | Constant power/current/impedance fractions for P and Q behavior. |
| dynamic_parameters | Voltage/frequency sensitivity, recovery constants, reconnect logic, and related dynamic fields. |
| load_composition | Electronic, motor, VFD, and static fractions or equivalent composition indicators. |
| ramping | Ramp up/down limits, staging behavior, and restoration pickup characteristics where available. |
| generator_interactions | Islanding, export, transition, and constraint flags associated with backup generation. |
| power_quality | Harmonic risk, screening results, and study recommendations. |
| confidence_tags | Confidence designation for each parameter or parameter group. |
| provenance | Evidence, rule, or assumption references tied to each output parameter. |

## 6.2 Scenario Requirements

ScenarioSet must produce Typical, Conservative, and Best-Case variants when uncertainty exists.

Scenario outputs must preserve provenance and add scenario rationale where applicable.

Scenario labels and bounds must remain explicit in exported artifacts.

# 7. Provenance and Confidence

Every OutputParameter must include provenance referencing an EvidenceSnippet, deterministic rule identifier, or explicit assumption.

Confidence levels should distinguish direct specification support, class-based inference, and assumption-driven estimates.

Export must fail closed if required provenance is missing.

# 8. Implementation Notes

Ingestion, search, and retrieval may be implemented using enterprise search platforms (for example IBM Watson Discovery).

Model-assisted extraction or translation may be implemented with foundation model services (for example watsonx.ai).

These references describe possible implementations and do not change the platform-agnostic schema design.